2019 TRIUMPH `MODERN CLASSICS’ SPECIFICATIONS

|  | STREET TWIN | STREET SCRAMBLER |
| --- | --- | --- |
| Engine Type | Liquid cooled, 8 valve, SOHC, 270° crank angle parallel twin | Liquid cooled, 8 valve, SOHC, 270° crank angle parallel twin |
| Capacity | 900cc | 900cc |
| Bore/Stroke | 84.6 x 80 mm | 84.6 x 80 mm |
| Compression Ratio | 11.0: 1 | 11.0: 1 |
| Maximum Power | 65PS / 64 BHP (48 kW) @ 7,500 rpm | 65PS / 64 BHP (48 kW) @ 7,500 rpm |
| Maximum Torque | 80 Nm @ 3,200 rpm | 80 Nm @ 3,200 rpm |
| Fuel system | Multipoint sequential electronic fuel injection | Ride by wire, Multipoint sequential electronic fuel injection |
| Exhaust | Brushed 2 into 2 exhaust system with twin brush silencers | Brushed 2 into 2 exhaust system with twin high-level brushed silencers |
| Final drive | O ring chain | O ring chain |
| Clutch | Wet, multi-plate assist clutch, cable operated | Wet, multi-plate assist clutch, cable operated |
| Gearbox | 5-speed | 5-speed |
| Frame | Tubular steel cradle | Tubular steel twin cradle |
| Swingarm | Twin-sided, tubular steel | Twin-sided, steel fabrication |
| Front Wheel | Cast aluminium alloy multi-spoke 18 x 2.75in | Spoked steel rims, 19 x 2.75in |
| Rear Wheel | Cast aluminium alloy multi-spoke 17 x 4.25in | Spoked steel rims, 17 x 4.25in |
| Front Tyre | 100/90-18 | 100/90 R19 |
| Rear Tyre | 150/70 R17 | 150/70 R17 |
| Front Suspension | KYB 41mm cartridge forks with cartridge damping, 120mm travel | KYB 41mm forks with cartridge damping, 120mm travel |
| Rear Suspension | KYB twin shocks with adjustable preload, 120mm rear wheel travel | KYB twin shocks with adjustable preload, 120mm rear wheel travel |
| Front Brake | Single 310mm disc, Brembo 4-piston fixed calipers, ABS | Single 310mm disc, Brembo 4-piston fixed calipers, ABS |
| Rear Brake | Single 255mm disc, Nissin 2-piston floating caliper, ABS | Single 255mm disc, Nissin 2-piston floating caliper, ABS |
| Width (Handlebars) | 785 mm | 835 mm |
| Height Without Mirrors | 1,114 mm | 1,180 mm |
| Seat Height | 760 mm | 790 mm |
| Wheelbase | 1,415 mm | 1,445 mm |
| Rake | 25.1 º | 25.6 º |
| Trail | 102.4 mm | 109 mm |
| Dry Weight | 198 Kg | 198 Kg |
| Fuel Tank Capacity | 12.0 L | 12.0 L |
| Fuel Consumption | 3.9/100km (72.4 mpg) | 4.1 l/100km (68.9 mpg) |
| CO2 Emissions | 89.0 g/km | 93.0 g/km |

|  | SCRAMBLER 1200 XC | SCRAMBLER 1200 XE |
| --- | --- | --- |
| Engine Type | Liquid cooled, 8-valve, SOHC, 270° crank angle parallel twin | Liquid cooled, 8-valve, SOHC, 270° crank angle parallel twin |
| Capacity | 1,200cc | 1,200cc |
| Bore/Stroke | 97.6 x 80 mm | 97.6 x 80 mm |
| Compression Ratio | 11.0: 1 | 11.0: 1 |
| Maximum Power | 90 PS / 89 Bhp (66.2 kW) @ 7,400 rpm | 90 PS / 89 Bhp (66.2 kW) @ 7,400 rpm |
| Maximum Torque | 110 Nm @ 3,950 rpm | 110 Nm @ 3,950 rpm |
| Fuel system | Multipoint sequential electronic fuel injection | Multipoint sequential electronic fuel injection |
| Exhaust | Brushed 2 into 2 exhaust system with twin silencers | Brushed 2 into 2 exhaust system with twin silencers |
| Final drive | X ring chain | X ring chain |
| Clutch | Wet, multi-plate assist clutch | Wet, multi-plate assist clutch |
| Gearbox | 6-speed | 6-speed |
| Frame | Tubular steel with aluminium cradle | Tubular steel with aluminium cradle |
| Swingarm | Twin-sided, aluminium, 547mm long | Twin-sided, aluminium, 579mm long |
| Front Wheel | Tubeless 36-spoke 21 x 2.15in, aluminium rims | Tubeless 36-spoke 21 x 2.15in, aluminium rims |
| Rear Wheel | Tubeless 32-spoke 17 x 4.25in, aluminium rims | Tubeless 32-spoke 17 x 4.25in, aluminium rims |
| Front Tyre | 90/90-21 | 90/90-21 |
| Rear Tyre | 150/70 R17 | 150/70 R17 |
| Front Suspension | Showa ⌀45mm fully adjustable USD forks. 200mm wheel travel | Showa ⌀47mm fully adjustable USD forks. 250mm wheel travel |
| Rear Suspension | Öhlins fully adjustable piggy-back RSUs with twin springs. 200mm wheel travel | Öhlins fully adjustable piggy-back RSUs with twin springs. 250mm wheel travel |
| Front Brake | Twin 320mm discs, Brembo M50 monoblock calipers, radial master cylinder. Switchable ABS | Twin 320mm discs, Brembo M50 monoblock calipers, radial master cylinder. Switchable Cornering ABS |
| Rear Brake | Single 255mm disc, Brembo 2- piston floating caliper. Switchable ABS | Single 255mm disc, Brembo 2- piston floating caliper. Switchable Cornering ABS |
| Width (Handlebars) | 840 mm | 905 mm |
| Height Without Mirrors | 1,200 mm | 1,250 mm |
| Seat Height | 840 mm | 870 mm |
| Wheelbase | 1,530 mm | 1,570 mm |
| Rake | 25.8 º | 26.9 º |
| Trail | 121 mm | 129.2 mm |
| Dry Weight | 205 Kg | 207 Kg |
| Fuel Tank Capacity | 16 L | 16 L |
| Fuel Consumption | 4.9 l/100km (58 mpg) | 4.9 l/100km (58 mpg) |
| Emissions | 113.0 g/km | 113.0 g/km |

|  | SPEED TWIN |
| --- | --- |
| Engine Type | Liquid cooled, 8 valve, SOHC, 270° crank angle parallel twin |
| Capacity | 1,200cc |
| Bore/Stroke | 97.6 x 80 mm |
| Compression Ratio | 11.0: 1 |
| Maximum Power | 97 PS / 96 BHP (71.5 kW) @ 6,750 rpm |
| Maximum Torque | 112 Nm @ 4,950 rpm |
| Fuel system | Multipoint sequential electronic fuel injection |
| Exhaust | Brushed 2 into 2 exhaust system with twin silencers |
| Final drive | Chain |
| Clutch | Wet, multi-plate assist clutch |
| Gearbox | 6-speed |
| Frame | Tubular steel with aluminium cradle |
| Swingarm | Twin-sided, aluminium |
| Front Wheel | Cast aluminium alloy 7-spoke 17 x 3.5in |
| Rear Wheel | Cast aluminium alloy 7-spoke 17 x 5in |
| Front Tyre | 120/70 ZR17 |
| Rear Tyre | 160/60 ZR17 |
| Front Suspension | 41mm cartridge forks, 120mm travel |
| Rear Suspension | Twin shocks with adjustable preload, 120mm rear wheel travel |
| Front Brake | Twin 305mm discs, Brembo 4-piston fixed calipers, ABS |
| Rear Brake | Single 220mm disc, Nissin 2-piston floating caliper, ABS |
| Width (Handlebars) | 760 mm |
| Height Without Mirrors | 1,110 mm |
| Seat Height | 807 mm |
| Wheelbase | 1,430 mm |
| Rake | 22.8 º |
| Trail | 93.5 mm |
| Dry Weight | 196 Kg |
| Fuel Tank Capacity | 14.5 L |
| Fuel Consumption | 4.8 l/100km (59 mpg) |
| CO2 Emissions | 109.0 g/km |